1.TRAFFIC LIGHT

source code


// FSM-Based Traffic Light Controller

// Arduino UNO - Proteus 8 Professional

#define NS_RED     2

#define NS_YELLOW  3

#define NS_GREEN   4

#define EW_RED     5

#define EW_YELLOW  6

#define EW_GREEN   7

void setup()

{

pinMode(NS_RED, OUTPUT);

pinMode(NS_YELLOW, OUTPUT);

pinMode(NS_GREEN, OUTPUT);

pinMode(EW_RED, OUTPUT);

pinMode(EW_YELLOW, OUTPUT);

pinMode(EW_GREEN, OUTPUT);

Serial.begin(9600);

Serial.println("FSM TRAFFIC LIGHT CONTROLLER");

}

void loop()

{

// STATE S0: North-South GREEN

digitalWrite(NS_RED, LOW);

digitalWrite(NS_YELLOW, LOW);

digitalWrite(NS_GREEN, HIGH);

digitalWrite(EW_RED, HIGH);

digitalWrite(EW_YELLOW, LOW);

digitalWrite(EW_GREEN, LOW);

Serial.println("S0: NS GREEN - EW RED");

delay(10000);

// STATE S1: North-South YELLOW

digitalWrite(NS_GREEN, LOW);

digitalWrite(NS_YELLOW, HIGH);

Serial.println("S1: NS YELLOW - EW RED");

delay(3000);

// STATE S2: East-West GREEN

digitalWrite(NS_YELLOW, LOW);

digitalWrite(NS_RED, HIGH);

digitalWrite(EW_RED, LOW);

digitalWrite(EW_GREEN, HIGH);

Serial.println("S2: NS RED - EW GREEN");

delay(10000);

// STATE S3: East-West YELLOW

digitalWrite(EW_GREEN, LOW);

digitalWrite(EW_YELLOW, HIGH);

Serial.println("S3: NS RED - EW YELLOW");

delay(3000);

digitalWrite(EW_YELLOW, LOW);

}

2.HEALTH MONITORING

SOURCE CODE :

#include <Wire.h>

#define LM75_ADDRESS 0x48

#define PULSE_PIN 8

unsigned long pulseCount = 0;

unsigned long lastTempTime = 0;

unsigned long lastBpmTime = 0;

int previousPulseState = LOW;

float readTemperature()

{

Wire.beginTransmission(LM75_ADDRESS);

Wire.write(0x00);

Wire.endTransmission();

Wire.requestFrom(LM75_ADDRESS, 2);

if (Wire.available() >= 2)

{

byte msb = Wire.read();

byte lsb = Wire.read();

int16_t rawTemperature =

((int16_t)msb << 8) | lsb;

return rawTemperature / 256.0;

}

return -999.0;

}

void setup()

{

Serial.begin(9600);

Wire.begin();

pinMode(PULSE_PIN, INPUT);

Serial.println("I2C HEALTH MONITORING NODE");

Serial.println("---------------------------");

}

void loop()

{

unsigned long currentTime = millis();

// Pulse counting

int currentPulseState = digitalRead(PULSE_PIN);

if (currentPulseState == HIGH &&

previousPulseState == LOW)

{

pulseCount++;

}

previousPulseState = currentPulseState;

// Temperature acquisition every 1 second

if (currentTime - lastTempTime >= 1000)

{

lastTempTime = currentTime;

float temperature = readTemperature();

Serial.print("Temperature: ");

if (temperature > -100)

{

Serial.print(temperature, 1);

Serial.println(" C");

}

else

{

Serial.println("Sensor Error");

}

}

// BPM calculation every 5 seconds

if (currentTime - lastBpmTime >= 5000)

{

lastBpmTime = currentTime;

float bpm = (pulseCount * 60.0) / 5.0;

Serial.print("Pulse Rate: ");

Serial.print(bpm, 0);

Serial.println(" BPM");

Serial.println("---------------------------");

pulseCount = 0;

}

}